# Wheeler Lab Protocols

## Objective

Provide detailed procedures for DNA extraction using the Qiagen DNeasy kit, PCR amplification using Ready-to-go Beads, agarose gel electrophoresis, gel extraction for double bands, AMPure cleanup, sequencing setup, and CleanSeq cleanup prior to sequencing.

## Materials

Qiagen DNeasy kit buffers: Buffer ATL, Proteinase K, Buffer AL, 100% Ethanol, Buffer AW1, Buffer AW2, Buffer AE

UV-crosslinked 1.5 mL tubes

Fine Science Tools heat block (≥200 °C)

Centrifuge capable of 8000 rpm and 13000 rpm

PCR reagents: Ready-to-go Beads, primers (10 µM), H2O

Agarose

1X TBE buffer

SYBRsafe stain

Loading dye

Promega Wizard Kit reagents: low melting point agarose, ethidium bromide, Membrane Binding Solution, Membrane Wash Solution, nuclease-free water

AMPure beads

Big Dye Extender Buffer

Big Dye primer (3.2 µM)

CleanSeq reagents

85% Ethanol

0.5 mM EDTA

Eppendorf plates and tubes

ABI 3730 sequencer

## Procedure

### DNA Extraction – Qiagen DNeasy kit

Isolate a suitable piece of tissue and place it in a UV-crosslinked 1.5 mL tube.

Add 180 µL Buffer ATL and 20 µL Proteinase K to the tube and vortex.

Incubate the tube at 55 °C for 3 hours or overnight.

Remove the tube from the incubator, vortex briefly, then add 200 µL Buffer AL and vortex again.

Place the tube in a 70 °C heat block for 10 minutes.

Add 200 µL 100% Ethanol to the tube and transfer the entire volume onto a spin column.

Centrifuge the spin column at 8000 rpm for 1 minute and discard the flow-through.

Add 500 µL Buffer AW1 to the spin column and centrifuge at 8000 rpm for 1 minute; discard the flow-through.

Add 500 µL Buffer AW2 to the spin column and centrifuge at 13000 rpm for 3 minutes; discard the flow-through.

Place the spin column in a UV-crosslinked 1.5 mL tube, add 200 µL Buffer AE, and let it sit for 1 minute.

Centrifuge the spin column at 8000 rpm for 1 minute.

Repeat the elution with 200 µL Buffer AE and combine both flow-throughs for a total volume of 400 µL.

### PCR – Ready-to-go Beads

Prepare the PCR mix as follows:

2 µL DNA template

21 µL H2O

1 µL forward primer (10 µM)

1 µL reverse primer (10 µM)

Seal the PCR tubes or plate with caps or wax.

Use one of the following thermal profiles:

**Basic program:**

94 °C for 5:00 minutes

35 cycles of:

94 °C for 0:30 seconds

50 °C for 0:30 seconds (annealing time can be increased to 1:00 minute)

72 °C for 0:45 seconds (extension can be increased up to 1:30 for longer genes)

72 °C for 7:00 minutes

Hold at 10 °C

**Rock and Roll:**

94 °C for 5:00 minutes

40 cycles of:

94 °C for 0:15 seconds

42 °C for 0:05 seconds (can increase annealing time up to 0:30 seconds)

68 °C for 0:15 seconds (can increase extension time up to 0:30 seconds)

72 °C for 7:00 minutes

Hold at 10 °C

**Down/Up:**

94 °C for 5:00 minutes

10 cycles at each of the following:

94 °C for 0:30 seconds

Annealing at 54 °C, 50 °C, 52 °C, 54 °C for 0:45 seconds each

72 °C for 1:30 minutes

72 °C for 7:00 minutes

Hold at 10 °C

Use recommended annealing temperatures for specific primers:

12s: basic program at 50 °C or 48 °C (minimum 46 °C)

16s, 18s, 28s: basic program at 50 °C or 48 °C

COI: Rock and roll starting at 48 °C, can go as low as 42 °C

ExtA/B: Rock and roll with longer annealing/extension

Histone3: basic program at 50 °C or 52 °C

EF-1 alpha: basic program with longer annealing and extension at 54 °C or 56 °C or using Down/Up program

### Agarose Gel Electrophoresis

Prepare 400 mL of 1X TBE buffer and add 8-10 g agarose.

Microwave the mixture for 5 minutes, then pour into gel tray and let cool for 30 minutes.

Add SYBRsafe stain (0.1 µL per lane) to 3 µL of loading dye; add SYBRsafe stain similarly to the DNA ladder.

Mix 2 µL of PCR product with 3 µL of the loading dye and load on the gel to check amplifications.

If double bands appear consistently, perform gel extraction.

If faint bands appear, clean using AMPure beads then concentrate the product.

### Gel Extraction (for double bands) – Promega Wizard Kit

Prepare a 1% low melting point agarose gel by dissolving 0.5 g agarose in 50 mL 1X TBE buffer and adding 1 µL ethidium bromide.

Add 5 µL loading dye to your PCR product and load the entire volume onto the gel.

Run the gel at 70 volts for 1 to 1.5 hours.

Using UV protection and a razor blade or spatula, excise the desired band and place it in a 1.5 mL tube.

Weigh the gel slice and add 10 µL Membrane Binding Solution per 10 mg of gel.

Incubate at 55 °C for 10 minutes or until the gel is dissolved.

Transfer the entire volume onto a minicolumn and let sit for 1 minute.

Centrifuge the minicolumn at 13000 rpm for 1 minute and discard flow-through.

Add 700 µL Membrane Wash Solution to the minicolumn.

Centrifuge at 13000 rpm for 1 minute and discard flow-through.

Add 500 µL Membrane Wash Solution.

Centrifuge at 13000 rpm for 5 minutes and discard flow-through.

Re-centrifuge for 1 minute with lid open.

Transfer minicolumn to a new 1.5 mL tube.

Add 50 µL nuclease-free water and let sit for 1 minute.

Centrifuge at 13000 rpm for 1 minute.

Use the eluted DNA directly for sequencing or re-amplify using a regular PCR protocol to concentrate the product.

### AMPure Cleanup

Transfer 18 µL AMPure beads to an Eppendorf plate.

Add 10 µL PCR product and mix thoroughly.

Let the mixture sit for 5 minutes.

Place the plate on a magnet and let sit for 5-10 minutes.

Remove the entire liquid volume, leaving the beads; discard the removed liquid.

Add 200 µL 70% Ethanol while on the magnet, let sit for 30 seconds, then flip plate to remove ethanol.

Repeat the ethanol wash once.

Remove the plate from the magnet and let air dry for 12-25 minutes.

Add 40 µL H2O and mix well to resuspend the DNA.

When ready for sequencing reaction setup, place the plate back on the magnet for 10 minutes.

**Faint band concentration:**

After AMPure cleanup, place the plate on the magnet for 10 minutes and transfer the product to a new tube or plate.

Evaporate to dryness using a DNA speedvac.

Resuspend the dried product in 10 µL H2O and proceed to sequencing reactions.

### Sequencing Setup

Prepare the sequencing mix in an Eppendorf plate as follows:

5 µL cleaned PCR product

1 µL Big Dye Extender Buffer

1 µL 3.2 µM primer

1 µL Big Dye

Seal the plate with wax or heat seal.

Use the following thermal profile:

94 °C for 0:30 seconds

35 cycles of:

50 °C for 1:00 minute

60 °C for 4:00 minutes

Hold at 4 °C

### CleanSeq Cleanup

Add 10 µL CleanSeq directly to the sequencing reaction product.

Add 37.5 µL 85% Ethanol and mix.

Place the plate on a magnet for 5 minutes.

Flip the plate while still on the magnet and discard ethanol.

Add 100 µL 85% Ethanol and let sit for 30 seconds.

Flip the plate again on the magnet and discard ethanol.

Remove the plate from the magnet and let air dry for 10 minutes.

Add 40 µL 0.5 mM EDTA and let sit for 5 minutes.

Place the plate back on the magnet for 5 minutes.

Transfer 33 µL of the finished product to an optical or Eppendorf plate, briefly spin down to remove bubbles, and heat seal.

Run the plate on an ABI 3730 sequencer.

## Review Checklist

[ ] Growth conditions verified (temperature, CO2/O2, time)

[ ] Media composition and volumes verified

[ ] Mixing parameters verified (rpm/RCF, time)

[ ] Incubation times verified

[ ] Centrifugation settings verified (RCF, time, temp)

[ ] Step order confirmed against source

[ ] Any ambiguous values flagged with [CHECK]

## Review Flags

## Critical Parameters Checklist (from the protocol)

Temperatures:

DNA extraction: 55 °C (3 h or overnight), 70 °C (10 min), heat block ≥200 °C (sterilization)

PCR thermal profiles: 94 °C, 50 °C, 42 °C, 54 °C, 56 °C, 60 °C, 68 °C (various steps and times)

Gel extraction incubation: 55 °C (10 min)

Times:

DNA extraction incubation: 3 h or overnight; Buffer AL incubation 10 min; centrifugations 1 or 3 min

PCR steps: denaturation 5 min initial, cycles 30 s to 1 min annealing, extension up to 1:30 min, final extension 7 min

Gel cooling: 30 min

Gel electrophoresis run: 1–1.5 h

AMPure bead incubations: 5 min, ethanol washes 30 s

CleanSeq: magnet times 5 min, drying 10 min, EDTA incubation 5 min

Volumes:

DNA extraction buffers: 180 µL Buffer ATL, 20 µL Proteinase K, 200 µL Buffer AL, 200 µL Ethanol, 500 µL AW1, 500 µL AW2, 200 µL AE (twice)

PCR mix: 2 µL DNA, 21 µL water, 1 µL each primer (10 µM)

Agarose gel: 8–10 g in 400 mL buffer

Gel extraction: 0.5 g agarose in 50 mL + 1 µL EtBr, 5 µL loading dye, 10 µL Membrane Binding Solution/10 mg gel, 700 µL and 500 µL Wash Solutions, 50 µL elution

AMPure: 18 µL beads, 10 µL PCR product, 200 µL 70% ethanol

Sequencing setup: 5 µL cleaned PCR product, 1 µL Big Dye Extender Buffer, 1 µL primer (3.2 µM), 1 µL Big Dye

CleanSeq: 10 µL CleanSeq, 37.5 µL 85% ethanol, 100 µL 85% ethanol, 40 µL 0.5 mM EDTA, 33 µL final product

Centrifugation:

DNA extraction: 8000 rpm (1 min), 13000 rpm (1 or 3 min)

Gel extraction: 13000 rpm (1, 5, and 1 min steps)

Other:

Forceps sterilization: ≥200 °C for ≥20 s

Gel stain: 0.1 µL SYBRsafe per lane

Electrophoresis: 70 V for 1–1.5 h

---

## Potential Missing Parameters (compare to source)

Proteinase K volume was typed as "20.L" in source and protocol uses "20 µL"—consistent, no issue.

The centrifuge capacity "up to 30 samples" noted in source is not mentioned in the protocol. [CHECK]

The source states "vortex" after adding Buffer ATL/proteinase K and after additions of Buffer AL; this is presented but vortex durations/intensity missing. [CHECK]

Source states "Use UV-crosslinked 1.5mL tubes" for placing tissue and spin column elution; protocol mentions UV-crosslinked tubes but does not repeat this at every relevant step. [CHECK]

PCR primer concentration in source: "10.M" likely 10 µM but not corrected in source; protocol states 10 µM—assumed correct.

Source mentions adding wax or caps to PCR tubes; protocol includes caps or wax.

PCR thermal profiles all well-transferred; however, some source lines truncated (e.g., ExtA/B and EF-1 alpha descriptions incomplete); protocol completes them but this lacks full source coverage. [CHECK]

Agarose gel prep: source states 8-10 g in 400 mL TBE; protocol matches.

Gel stain usage: source says add SYBRsafe to loading dye and ladder; protocol mentions this but no concentration for ladder dye stain—[CHECK].

Gel extraction step 17: "Go directly to sequencing or re-amplify using a regular PCR protocol" is summarized without mentioning to concentrate product after re-amplification. [CHECK]

AMPure clean up notes "Robot or by hand" in source but no mention in protocol. [Not critical but noted]

Source mentions faint band concentration procedure using DNA speedvac; protocol includes it.

Sequencing setup: source has an ambiguous "1µL Big Dye" partially in text; protocol clarifies amount and components—assumed correct.

In sequencing thermal profile, source text is missing specific times for 94 °C denaturation per cycle and primer extension phases; protocol fills in times, but this is not clearly supported by the incomplete source text and may be interpolation. [CHECK]

---

## Possible Unsupported Claims

The protocol states "PCR primers 10 µM," confirming source misprint (10.M); assuming correction, but source does not clarify explicitly.

"Run on ABI 3730" in CleanSeq cleanup stated as last step; source states similarly.

The "Robot or by hand" option for AMPure and CleanSeq cleanup not elaborated in protocol beyond mention in Materials and Procedure titles.

"Forceps sterilized for ≥20 seconds" is in source; protocol states ≥200 °C but does not specify time ≥20 s. [CHECK]

Protocol claims "microwave for 5 minutes" for agarose gel prep; source states this exactly.

The Down/Up PCR program annealing at multiple temperatures with 10 cycles each is reproduced but source is partially cut off; completeness of protocol claim may exceed source text fidelity. [CHECK]

---

## Step Order / Omission Risks

Step order for DNA extraction matches source.

PCR setup and thermal profiles preserve source order and info.

Agarose gel prep steps match source order.

Gel extraction steps align with source.

AMPure cleanup and faint band concentration steps are complete and ordered.

Sequencing setup and CleanSeq cleanup steps follow source order.

The source mentions "rock and roll" and "down/up" PCR cycles without clarifying transitions between annealing temps/time; protocol approximates this but potential for ambiguity in cycle transitions. [CHECK]

Sterilization of forceps step is mentioned only in Materials and not explicitly as initial step before extraction; possible omission. [CHECK]

---

# Summary

Most numeric and procedural parameters from source are well represented in the generated protocol.

Minor clarifications and possible missing details include vortex durations, centrifuge capacity, forceps sterilization time, and primer concentration correction.

Some PCR program details partly extrapolated from truncated source text.

No major unsupported claims detected but some steps may benefit from flagged ambiguity.

Step orders consistent but sterilization step not explicitly placed before tissue isolation.

# Flags:

Vortex duration/intensity missing [CHECK]

Forceps sterilization duration missing [CHECK]

Centrifuge max sample load missing [CHECK]

Partial/incomplete PCR program details [CHECK]

Gel stain concentration for ladder unspecified [CHECK]

Sterilization step placement implicit/not explicit in protocol [CHECK]