# TRIzol™ Reagent Experimental protocol for DNA isolation

## Objective

Isolate high quality DNA from cell and tissue samples of human, animal, plant, yeast, or bacterial origin using TRIzol™ Reagent.

## Materials

TRIzol™ Reagent (100 mL or 200 mL)

Chloroform

Ethanol, 100%

Ethanol, 75%

0.1 M sodium citrate in 10% ethanol, pH 8.5

8 mM NaOH

HEPES

Polypropylene microcentrifuge tubes

Centrifuge capable of 12,000 × g at 4°C

Homogenizer (for tissue samples)

RNaseZap™ RNase Decontamination Solution (optional)

## Procedure

### Lyse samples and separate phases

Lyse and homogenize samples in TRIzol™ Reagent according to your starting material:

For tissues: Add 1 mL of TRIzol™ Reagent per 50–100 mg of tissue; homogenize using a homogenizer.

For cells grown in monolayer:

Remove growth media.

Add 0.3–0.4 mL of TRIzol™ Reagent per 1 × 10^5 to 1 × 10^7 cells directly to the culture dish to lyse the cells.

Pipet the lysate up and down several times to homogenize.

For cells grown in suspension:

Pellet the cells by centrifugation and discard the supernatant.

Add 0.75 mL of TRIzol™ Reagent per 0.25 mL of sample (5–10 × 10^6 cells from animal, plant, or yeast origin, or 1 × 10^7 cells of bacterial origin) to the pellet.

Incubate for 5 minutes to permit complete dissociation of the nucleoprotein complex.

Add 0.2 mL of chloroform per 1 mL of TRIzol™ Reagent used for lysis; securely cap the tube.

Incubate for 2–3 minutes at room temperature (20–25°C).

Centrifuge the sample for 15 minutes at 12,000 × g at 4°C.

The mixture separates into a lower red phenol-chloroform phase, an interphase, and a colorless upper aqueous phase.

Discard the aqueous phase containing the RNA. Proceed with the interphase and phenol-chloroform phase for DNA isolation.

### Isolate DNA

#### 1. Precipitate the DNA

Remove any remaining aqueous phase overlying the interphase carefully.

Add 0.3 mL of 100% ethanol per 1 mL of TRIzol™ Reagent used for lysis.

Cap the tube and mix by inverting several times.

Incubate for 2–3 minutes at room temperature.

Centrifuge for 5 minutes at 2000 × g at 4°C to pellet the DNA.

Transfer the phenol-ethanol supernatant to a new tube for protein isolation or storage at –70°C.

#### 2. Wash the DNA

Resuspend the pellet in 1 mL of 0.1 M sodium citrate in 10% ethanol, pH 8.5, per 1 mL of TRIzol™ Reagent used for lysis.

Incubate for 30 minutes at room temperature, mixing occasionally by gentle inversion.

Note: DNA can be stored in sodium citrate/ethanol for at least 2 hours at this stage.

Centrifuge for 5 minutes at 2000 × g at 4°C.

Discard the supernatant using a micropipettor.

Repeat the wash steps 2a–2d once (twice for large DNA pellets >200 µg).

Resuspend the pellet in 1.5–2 mL of 75% ethanol per 1 mL of TRIzol™ Reagent used.

Incubate for 10–20 minutes at room temperature with occasional gentle inversion.

Note: DNA can be stored in 75% ethanol for several months at 4°C.

Centrifuge for 5 minutes at 2000 × g at 4°C.

Discard the supernatant with a micropipettor.

Vacuum or air dry the DNA pellet for 5–10 minutes without using a vacuum centrifuge.

#### 3. Solubilize the DNA

Resuspend the pellet in 0.3–0.6 mL of 8 mM NaOH by pipetting up and down.

Note: Solubilization in mild base is preferred because DNA does not resuspend well in water or Tris buffer.

Centrifuge for 10 minutes at 12,000 × g at 4°C to remove insoluble materials.

Transfer the supernatant to a new tube.

Adjust pH as needed with HEPES.

Proceed to downstream applications or store DNA at 4°C overnight.

For long-term storage at –20°C, adjust pH to 7–8 with HEPES and add 1 mM EDTA.

### Determine DNA yield

Use absorbance method:

Dilute sample in water or buffer (pH >7.5).

Measure absorbance at 260 nm and 280 nm.

Calculate DNA concentration as A260 × 50 = µg DNA/mL.

Calculate A260/A280 ratio; a ratio ~1.8 indicates pure DNA.

Use fluorescence method:

Quantify double-stranded DNA using an appropriate Qubit™ or Quant-iT™ ds DNA Assay Kit.

Typical DNA yields from various starting materials:

Fibroblasts, 1 × 10^6 cells: 5–7 µg DNA

Cultured mammal cells, 1 × 10^6 cells: 5–7 µg DNA

Skeletal muscles and brain, 1 mg: 2–3 µg DNA

Placenta, 1 mg: 2–3 µg DNA

Liver, 1 mg: 3–4 µg DNA

Kidney, 1 mg: 3–4 µg DNA

## Notes

Perform all steps at room temperature (20–25°C) unless otherwise noted.

Use cold TRIzol™ Reagent for samples with high RNase content (e.g., spleen or pancreas).

Use sterile, RNase-free plastic ware and gloves to prevent RNase contamination.

Do not wash cells before addition of TRIzol™ Reagent to avoid mRNA degradation.

Samples can be stored at 4°C overnight or at –20°C for up to a year after lysis.

Do not dry DNA pellet using vacuum centrifuge.

DNA can be stored in sodium citrate/ethanol washing solutions or 75% ethanol for extended periods.

Handle all reagents following safety guidelines; wear appropriate PPE.

### Troubleshooting

Lower DNA yield may result from incomplete homogenization; ensure complete immersion and homogenization.

Insufficient DNA solubilization can be addressed by pipetting repeatedly and heating sample to 50–60°C.

Sample degradation is avoided by immediate processing or freezing after collection.

Contaminating aqueous phase can reduce DNA quality; remove it completely before DNA precipitation.

Insufficient washing of DNA pellet reduces purity; wash pellet adequately with sodium citrate in ethanol.

## Review Checklist

[ ] Growth conditions verified (temperature, CO2/O2, time)

[ ] Media composition and volumes verified

[ ] Mixing parameters verified (rpm/RCF, time)

[ ] Incubation times verified

[ ] Centrifugation settings verified (RCF, time, temp)

[ ] Step order confirmed against source

[ ] Any ambiguous values flagged with [CHECK]

## Review Flags

## Critical Parameters Checklist (from the protocol)

TRIzol™ Reagent volume per sample:

Tissue: 1 mL per 50–100 mg

Monolayer cells: 0.3–0.4 mL per 1 × 10^5–10^7 cells

Suspension cells: 0.75 mL per 0.25 mL sample

Incubation times:

Post-homogenization: 5 minutes

After chloroform addition: 2–3 minutes

DNA ethanol incubation: 2–3 minutes

Sodium citrate in ethanol wash: 30 minutes

75% ethanol wash: 10–20 minutes

DNA pellet drying: 5–10 minutes

Centrifugation settings:

Lysate (after chloroform): 12,000 × g, 15 min, 4°C

High fat sample centrifugation (optional): 12,000 × g, 5 min, 4–10°C (mentioned in source, missing in protocol)

DNA precipitation centrifugation: 2000 × g, 5 min, 4°C

DNA pellet washing centrifugation: 2000 × g, 5 min, 4°C

Solubilization centrifugation: 12,000 × g, 10 min, 4°C

Chloroform volume: 0.2 mL per 1 mL TRIzol™

Ethanol for DNA precipitation: 0.3 mL 100% ethanol per 1 mL TRIzol™

Sodium citrate wash solution: 1 mL 0.1 M sodium citrate in 10% ethanol per 1 mL TRIzol™

75% ethanol wash: 1.5–2 mL per 1 mL TRIzol™

DNA resuspension volume: 0.3–0.6 mL 8 mM NaOH

Temperature for incubations: room temperature (20–25°C) unless otherwise noted

Storage conditions: samples can be stored at 4°C overnight or –20°C up to one year after lysis; DNA stored in sodium citrate/ethanol for ≥2 hours; DNA in 75% ethanol several months at 4°C; long-term DNA storage at –20°C with adjusted pH and 1 mM EDTA

DNA yield expectations for various samples listed

## Potential Missing Parameters (compare to source)

Optional centrifugation step for high fat samples (5 min, 12,000 × g, 4–10°C) before phase separation is mentioned in source but omitted in protocol.

Source states use cold TRIzol™ Reagent for high RNase samples (spleen, pancreas), but protocol mentions this only lightly in notes, not emphasized.

Source advises against washing cells before adding TRIzol™ to avoid mRNA degradation; protocol includes this but could emphasize in a warning box or clearer alert for DNA isolation step.

Source notes “sample volume should not exceed 10% of TRIzol™ volume” for lysis; this detail is missing in protocol.

Source includes optional storage times and temperatures for intermediate steps (e.g., storage at 4°C overnight or –20°C up to 1 year for lysed samples); partially included but could be clearer regarding potential impact on yield/quality.

## Possible Unsupported Claims

None detected. All procedure steps and notes align well with the source text.

The protocol’s statement that DNA can be stored in 75% ethanol for several months at 4°C is supported by source.

Troubleshooting points included are all referenced in source text.

## Step Order / Omission Risks

Missing optional centrifugation for high fat samples prior to phase separation is a risk factor. This could affect phase clarity and yield if omitted.

Source text notes careful removal of aqueous phase over interphase is critical for DNA quality; protocol includes this but no extra emphasis or caution flag could be a risk.

No explicit mention in protocol to not dry DNA pellet by vacuum centrifuge beyond note line; a stronger warning could mitigate risk.

No mention of the initial brief centrifugation step to clear lysates with high fat content as a “pre-lysis prep.”

All other steps appear in correct order matching source text.

---

Overall, the generated protocol is comprehensive and consistent with the source text, but the optional pre-centrifugation of high fat samples should be explicitly included or flagged as [CHECK], and more explicit warnings about avoiding cell washing before lysis and maximum sample volume relative to TRIzol™ could reduce user risk.