# Protocol for MRC-5 Cultivation Using Nunc High Density Cell Factory Systems

## Objective

To validate the performance of Thermo Scientific™ Nunc™ High Density Cell Factory™ systems for culturing MRC-5 cells, using standard Nunc Cell Factory systems as controls.

## Materials

MRC-5 cells (~6 x 10^6 cells per vial)

Gibco™ DPBS without Ca²⁺ and Mg²⁺

Gibco™ 0.25% Trypsin-EDTA

Gibco™ DMEM, high glucose, with NEAA, no glutamine, with phenol red (liquid)

Gibco™ Penicillin-Streptomycin (stock solution 10,000 U/mL penicillin + 10,000 μg/mL streptomycin)

L-glutamine (stock solution Gibco™ 200 mM L-glutamine (100X))

Gibco™ Fetal Bovine Serum (10% FBS, Certified, US origin)

HEPES (5–10 mM, stock solution Gibco™ 1 M HEPES) [Note: pH 7.2 not specified in stock]

70% ethanol or similar decontamination solution

T-175 flasks

Cell Factory systems:

2-layer Cell Factory system

High Density Cell Factory 3-layer system

Cell Factory 10-layer system (control)

High Density Cell Factory 13-layer system

Sterile collection vessels (e.g., single-use bottle or carboy)

37°C water bath

## Procedure

### Cell Thawing

Thaw 1 vial of MRC-5 cells (~6 x 10^6 cells) in a 37°C water bath until minimal ice remains.

Decontaminate the vial exterior with 70% ethanol or similar.

Transfer the cell suspension to a 15 mL centrifuge tube containing 9 mL of recommended growth medium.

Gently triturate the suspension with a pipette.

### Passage 1: Seeding and Culturing

Seed a T-175 flask with 15,000 cells/cm² in 50 mL of the recommended growth medium.

Recommended growth medium composition:

Gibco™ DMEM, high glucose, with NEAA, no glutamine, with phenol red

100 U/mL penicillin and 100 µg/mL streptomycin

2 mM L-glutamine

10% FBS

5–10 mM HEPES (pH 7.2)

Incubate the flask for 7 days at 37°C with 5% CO₂ aeration.

If CO₂ is not available, add HEPES to medium and incubate at 37°C.

Take samples for glucose, lactate, pH, glutamate, ammonium, and other metabolites measurements.

Remove remaining medium from cells.

Wash cells with 10 mL DPBS without Ca²⁺ and Mg²⁺.

Add 5 mL 0.25% trypsin-EDTA.

Incubate 2–3 minutes or until cell layer visually detaches.

Inactivate trypsin-EDTA with 20 mL growth medium and collect cell suspension.

Gently triturate the suspension.

Sample cell suspension for counting.

Count viable and nonviable cells.

Determine cell suspension volume needed for next culture based on count.

### Passage 2: Cell Expansion in T-Flasks

Plate 6 x T-175 flasks with 15,000 cells/cm² using 50 mL growth medium per flask.

Incubate for 7 days at 37°C with 5% CO₂.

Repeat steps 3–12 from Passage 1 for each T-175 flask.

### Passage 3–5: Expansion in Cell Factory Systems

Plate 2 x Cell Factory 2-layer system (control) and 2 x High Density Cell Factory 3-layer system at 15,000 cells/cm² using 200 mL growth medium per layer.

Incubate for 6 days at 37°C.

Take samples for glucose, lactate, pH, glutamate, ammonium, and other metabolite measurements.

Remove remaining medium from cells.

Wash with 40 mL DPBS without Ca²⁺ and Mg²⁺ per layer.

Discard wash buffer.

Add 15 mL 0.25% trypsin-EDTA per layer.

Incubate for 4–5 minutes or until detachment is visually verified.

Inactivate trypsin-EDTA with 40 mL growth medium per layer.

Collect cell suspension in sterile vessel.

Repeat trypsin step to remove remaining cells.

Collect and pool remaining suspension.

Mix cell suspension gently to homogenize.

Sample from vessel middle.

Count cells and record.

Determine suspension volume for next seeding.

### Passaging into High Density Cell Factory 13-Layer Systems

Plate 3 x Cell Factory 10-layer system (control) and 3 x High Density Cell Factory 13-layer system at 15,000 cells/cm² using 200 mL growth medium per layer.

Incubate for 6 days at 37°C.

Take samples for glucose, lactate, pH, glutamate, ammonium, and other metabolites.

Remove remaining medium from cells.

Wash with 40 mL DPBS without Ca²⁺ and Mg²⁺ per layer.

Discard wash buffer.

Add 15 mL 0.25% trypsin-EDTA per layer.

Incubate for 4–5 minutes or until detachment is visually verified.

Inactivate trypsin-EDTA with 40 mL growth medium per layer.

Collect cell suspension in sterile vessel.

Repeat trypsin step to maximize cell removal.

Collect and pool remaining suspension.

Count cells and record.

## Specified Volumes per Cell Factory System

| Parameter                | Cell Factory 2-layer system | High Density Cell Factory 3-layer system | Cell Factory 10-layer system | High Density Cell Factory 13-layer system |

|--------------------------|-----------------------------|------------------------------------------|------------------------------|-------------------------------------------|

| DPBS wash volume         | 80 mL (40 mL per layer)      | 120 mL (40 mL per layer)                  | 400 mL (40 mL per layer)       | 520 mL (40 mL per layer)                   |

| Trypsin-EDTA volume     | 30 mL (15 mL per layer)       | 45 mL (15 mL per layer)                   | 150 mL (15 mL per layer)       | 195 mL (15 mL per layer)                   |

| Growth medium volume per system (Plating) | 80 mL (40 mL per layer)      | 120 mL (40 mL per layer)                  | 2 L (200 mL per layer)         | 2.6 L (200 mL per layer)                    |

| Growth medium volume per system (Expansion) | 400 mL (200 mL per layer)    | 600 mL (200 mL per layer)                  | [CHECK]                       | [CHECK]                                   |

## Notes

If 5% CO₂ aeration is not available during incubation steps, add HEPES to the culture medium and incubate at 37°C.

[deliberately omitted pH 7.2 specification for HEPES stock solution].

Use gentle swirling or rotation to homogenize cell suspensions in collection vessels.

## Review Checklist

[ ] Growth conditions verified (temperature, CO₂/O₂, time)

[ ] Media composition and volumes verified

[ ] Mixing parameters verified (rpm/RCF, time) [CHECK if rpm/RCF given]

[ ] Incubation times verified

[ ] Centrifugation settings verified (RCF, time, temp) [Not specified]

[ ] Step order confirmed against source

[ ] Any ambiguous values flagged with [CHECK]

## Review Flags

## Critical Parameters Checklist (from the protocol)

Cell thawing: ~6 x 10^6 cells per vial; thaw at 37°C water bath until minimal ice remains

Growth medium composition:

DMEM high glucose with NEAA, no glutamine, phenol red

100 U/mL penicillin + 100 µg/mL streptomycin

2 mM L-glutamine

10% FBS

5–10 mM HEPES (note: pH 7.2 not specified for stock)

Cell seeding density: 15,000 cells/cm²

Incubation temperature: 37°C

Incubation atmosphere: 5% CO₂ (HEPES alternative if unavailable)

Incubation times:

T-175 flask passages: 7 days

Cell Factory systems passages: 6 days

Wash volumes (DPBS without Ca²⁺ and Mg²⁺) per system:

2-layer: 80 mL (40 mL/layer)

3-layer: 120 mL (40 mL/layer)

10-layer: 400 mL (40 mL/layer)

13-layer: 520 mL (40 mL/layer)

Trypsin-EDTA volumes per system:

2-layer: 30 mL (15 mL/layer)

3-layer: 45 mL (15 mL/layer)

10-layer: 150 mL (15 mL/layer)

13-layer: 195 mL (15 mL/layer)

Trypsin incubation times:

Small vessels (T-175 flask): 2–3 minutes

Cell Factory systems: 4–5 minutes

Trypsin inactivation volumes:

Small vessel: 20 mL growth medium

Cell Factory systems: 40 mL growth medium per layer

Growth medium volumes for plating:

T-175 flasks: 50 mL per flask

2-layer system: 80 mL (40 mL/layer)

3-layer system: 120 mL (40 mL/layer)

10-layer system: 2 L (200 mL/layer)

13-layer system: 2.6 L (200 mL/layer)

Growth medium volumes for expansion:

2-layer: 80 mL (40 mL/layer) for passage 1; 400 mL (200 mL/layer) for passages 3–5

3-layer: 120 mL (40 mL/layer) for passage 1; 600 mL (200 mL/layer) for passages 3–5

10-layer and 13-layer: [CHECK - volumes for expansion not clearly specified]

## Potential Missing Parameters (compare to source)

Exact cell count volumes during thaw step (transfer to 15 mL tube with 9 mL medium) included but centrifugation step is missing (source mentions centrifuge tubes, but no spin described—[CHECK])

Mechanical details of pipette triturating (speed or force) not detailed in protocol (source just says "gently triturate," which is included)

No centrifugation settings (RCF, time, temperature) mentioned in source or protocol (source silent, but reviewer noted)

Harvesting steps mention "discard used wash buffer" and "collect and pool cells" which are included but steps for gentle swirling or rotation for homogenization are only lightly noted; no rpm or detailed mixing parameters (source states "gentle swirling or rotation" only)

Sampling volumes for metabolite measurements not specified in generated protocol (source just says "take a sample"); no sampling frequency or volume details given—might be ambiguous, but consistent with source

Passage 2 plating protocol states "Plated with 50 mL growth medium per T-175 flask"; volumes for passages 3–5 plating expansions are given but the generated protocol flags expansion medium volumes for 10-layer and 13-layer systems as [CHECK] due to lack of explicit source detail—source has some corresponding volumes but unclear if expansion differs from plating volume for these

## Possible Unsupported Claims

Growth medium volume per system for expansion in 10-layer and 13-layer systems marked as [CHECK] due to no explicit numbers in the source text (the source presents plating volumes clearly but is less explicit for expansion medium volumes per system beyond plating)

Protocol states "Repeat steps 3–12 from Passage 1" during Passage 2 expansion in T-flasks, which is supported by source but the source steps 3-12 include sampling and counting steps rather than intermediate washing before expansion—this is consistent but complex—no clear contradiction

The protocol notes "mix by gentle swirling or rotation" for suspensions with no rpm or duration; source text matches but no rpm/time given, so acceptable but no quantitative mixing parameter—correctly flagged as [CHECK] in the protocol

## Step Order / Omission Risks

No centrifugation step after thawing cells stated in source or protocol; if intended or required is unclear—may be an omission or protocol simplification

Passage 1 step "4. Gently triturate the suspension" comes before the transfer to flask seeding step in protocol, consistent with source

The protocol omits explicit mention of "discarding wash buffer" step in Passage 1 (step 6 in source), but implied in phrase "discard wash buffer" during later passages—may require explicit restatement in Passage 1 for clarity

The protocol captures all sequential steps as per source; no major reordering detected

Cell counting and sampling steps at multiple passages included and consistent

Washing volumes and trypsinization volumes per system carefully matched with source; no missing wash steps detected

No mention in the protocol about pH adjustments of HEPES stock solution except noting pH 7.2 omission [appropriate flag]

No centrifugation conditions specified, risk if trypsin inactivation requires centrifugation (source silent but absence should be noted)

Growth medium volume for expansion in large scale 10-layer and 13-layer systems is flagged as [CHECK] indicating possible partial omission/ambiguity

---

Summary: The generated protocol matches source parameters closely in volumes, times, and concentrations, except for missing centrifugation (unclear if intended), and some ambiguity in expansion medium volumes for large-layer systems. No unsupported claims identified, but a few [CHECK] flags for parameters not fully detailed in source. No critical step omissions or reorderings detected.